--------------------------------------------------------------------------------

--

--   Controller Counter Benchmark

--

-- Model Source: Chip Level modelling with VHDL by Jim Armstrong

--

-- Testpattern Source : Sanjiv Narayan And Frank Vahid

--                      University of California, Irvine.

--

-- author: Champaka Ramachandran

--         University Of California, Irvine, CA 92717

--         champaka@balboa.eng.uci.edu

--

-- Written on Aug24th 1992

--

--------------------------------------------------------------------------------

-----------------

Functionality :

-----------------

The Armstrong counter counts up or down and the counting is stops when

limits are reached. It operates with the Clock and Strobe signals acting

as triggers. The signal CON termines the operation mode of the counter.

When CON=00, the counter is being cleared. When CON=01, the Limit gets

loaded. When CON=10 a count up is performed. When CON=11 a count down is

performed.

The counting is done when a positive edge of clock is observed. The decoding

is perfomed on the postive edge of the strobe signal and the limit is loaded

on a negative edge of Strobe signal.

Counting up or down is done only if the count enable or the CNTE signal is

high. This in turn is set by the rising edge of EN. EN is the signal which

determines if the counter has reached the limit value.  The signal EN is

changed either when the value CNT changes and the limit has to be checked,

or the direction of counting has been changed and the ENIT signal is enabled.

The ENIT signal determines that the decoder has been executed and the direction

of counting is likely to have changed. The ENIT signal is then reset after

EN has been enabled back in the decoder by the RENIT signal.

In case the counter reaches the limit, further counting is disabled till a

new limit is loaded or the counter is cleared.

----------

Anomalies :

----------

The model does not support the behaviour of the counter under these conditions :

1. The counter is counting towards the limit and the counting direction changes.

This means the counting should stop since the value now has crossed the

limit in the direction of count. But, the model does not support this behaviour.

For eg. The counter is at 1 and is counting up and the limit has been set at 7.

If the count direction is now changed to downward count, the counter has already

crossed the limit 7 during count down, so, it should stop counting.

2. The counter is counting towards the limit and the limit changes so that the

counter now exceeds the limit. This means the counting should stop since the

value now has crossed the limit in the direction of count. But, the model does

not support this behaviour.

For eg. The counter is at 7 and is counting up and the limit has been set at 14.

If the limit is now changed to 5, the counter has already crossed the limit 5

during count down, so, it should stop counting.