-------------------------------------------------------

--

--               TESTING STRATEGY

--

-- Author : D. Sreenivasa Rao

--          University of california, IRvine, CA, 92717.

--          dsr@balboa.eng.uci.edu, (714)856-5106.

--

-- Written 12 September, 1992.

--

-------------------------------------------------------

The behavior of the elliptic filter can be tested by applying

a set of test patterns that are fairly sufficient to perform

a "sanity check" on the various operators of the filter. In

case of digital filters, any input (along with a set of current

state vectors) will exercise all the operations of the filter

(since there are no conditional branches in the design). However,

a few different test patterns will exercise the design more

thoroughly, and we have presented in the file "test_vectors.vhdl"

a handful of such test vectors. The vectors were derived heuristically,

according to standard practices used in such cases. We consider

vectors that are all similar (say all 0's and all 1's), vectors that

have different combinations of even and odd numbers as current state

vectors, and vectors that are powers of 2.

The functional testing of the elliptic filter is usually done

witha  'delta' function, the rough approximation of which in the

digital domain is a vector that is '1' in the first instance, and is

'0' for all other states. The corresponding output has to be tested

by substituting the vector in the z-transform of the state equation.

This is represented by Pattern #1, followed by Patterns 2, 3, and 4.